Module 8: CCNA - VLANs and Inter-VLAN Routing

- Beginner Question

- Explain Switch?

Ans: Switch

Switch is a networking and communication device which is used to connect multiple devices in a network. It works using MAC address. With the help of switch, devices can communicate with each other and share resources like files, folders, printers, images, documents, and internet connection.

Switch sends data only to the required device, so network traffic is reduced and communication becomes faster.

There are two types of switches:

- Layer-2 Switch

- Layer-3 Switch (Multi-Layer Switch)

- Explain Switch Boot Sequence?

Ans:

Switch Boot Sequence

When a switch is powered ON, it follows a boot sequence to load the operating system and start working properly.

- POST

Post stands for Power-On Self-Test.

First, the switch performs POST to check whether the hardware components like CPU, RAM, ports, and interfaces are working correctly or not.
If any hardware problem is found, the switch shows an error message.

- ROM

ROM stands for Read Only Memory.

After POST, the switch checks the ROM.
ROM contains the bootstrap program and basic instructions required to start the switch. The bootstrap program helps the switch to load the IOS.

3. Flash Memory

Next, the switch searches the Flash memory.
Flash stores the Cisco IOS (Operating System).
The IOS file is loaded from Flash into RAM so that the switch can operate.

- RAM

RAM stands for Random Access Memory.

RAM is the temporary working memory of the switch.
After the IOS is loaded, the running configuration file and routing/switching processes are stored in RAM.
All current operations are performed here.
Data in RAM is lost when the switch is turned OFF.

- NVRAM

NVRAM also known as Non-Volatile RAM.

Finally, the switch checks NVRAM to load the startup configuration file.
This file contains saved configurations like VLANs, passwords, IP addresses, etc. Since NVRAM is non-volatile, the data remains saved even after power is OFF.

The boot sequence of a switch is:

POST → ROM → Flash → RAM → NVRAM

After completing all these steps, the switch becomes ready to forward data in the network.

3.   Explain Three Methods to access Switch Command Line Interface?

There are three methods to access the Switch Command Line Interface (CLI):

- Console Access
In this method, a console cable is used to connect PC/laptop directly to the switch console port. It is mainly used for initial configuration.

- Telnet Access
Telnet is used to access the switch remotely through network using IP address. It is less secure because data is transferred in plain text.

- SSH (Secure Shell) Access
SSH is also used for remote access, but it is more secure because it encrypts the data during communication.

  - Explain and Configuring the Cisco Internet Operating System?

Ans:

Basic Configurations

Switch

Switch> enable

Switch# configure terminal

Switch(config)# interface FastEthernet 0/0

Switch(config-if)# ip address 192.168.1.1 255.255.255.0

Switch(config-if)# no shutdown

Switch(config-if)#exit

Router

Router> enable

Router# configure terminal

Router(config)# interface serial 0/0/0

Router(config-if)# ip address 10.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface FastEthernet 0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

5 Explain Switch Port?

Ans: Switch Port

A switch port is a port present on the switch that is used to connect devices like computers, printers, routers, and other switches using Ethernet cable. The switch receives data from one port and sends it to the correct destination port using MAC addresses.

There are two types of Switch Ports are there:

1. Access Port

Access port is used to connect end devices such as PC and printer.
It carries traffic of only one VLAN.

2. Trunk Port

Trunk port is used to connect switches or routers.
It carries traffic of multiple VLANs.

Functions of Switch Port

- Connects devices to the network

- Transfers data between devices

- Helps in communication inside the network

- Supports VLAN communication

- R1, R2, R3, and R4 have their Fast Ethernet 0/0 interfaces attached to the same VLAN. A network engineer has typed a configuration for each router by using a word processor. He will later copy and paste the configuration into the routers. Examine the following exhibit, which lists configuration for the four routers, as typed by the network engineer. Assuming that all four routers can ping each other’s LAN IP addresses after the configuration has been applied, choose the routers that will be able to form a neighbor relationship with the other routers on the LAN. (You can assume that, if not shown in the exhibit, all other related parameters are still set to their defaults.) (Choose two)

Ans:

2.  A. R1

B. R2

3.    A. MD5

4.   D. Router 2.2.2.2 is a backup designated router.

5.   D. netsh interface ipv6 show neighbors

6.   A. List

7.   B. Time to Live

C. Source address

D. Destination address